DMREFConceptOutline—Critical-Mineral-LeanPrintableAluminumAlloysviaCALPHAD–BOFN DiscoveryandUltrasonicAtomization

PI:SterlingBaird(BrighamYoungUniversity,MaterialsScience&Engineering).Solicitation:NSFDMREF(NSF25-508).Type: 4-yr,multi-PI,∼$1.5–2.0M.Anticipatedcycle:Jan–Feb2027.Keywords:MGI,BOFN,CALPHAD,LPBF,ultrasonicatomization, criticalmaterials.

| Feedstock | CALPHAD | pKGFNBOFN | Atomization | LPBF | PhaseID+Test |
| --- | --- | --- | --- | --- | --- |
| wire/scrap/powder | pycalphad/TC-Python/AMModule | multi-fidelityinference;GAinner | UA(BYU) | coupon | SEM/TEM,tensile |

closed-loopfeedback:phaseID+propertyupdateCALPHADpriorsandBOFNposterior

Fundamentalquestion

Whatcontrolstheprintabilityandelevated-temperaturephasestabilityofdilutemulti-principal-elementAlalloysbuiltfrom Mn/Cr/Zr/Si/Feadditions,intheregimewhereconventionalCALPHADassessmentsaresparseandrapid-solidification AMthermalhistoriesdrivenon-equilibriumphaseselection?TheMGI-relevantgapisnotoptimizationofanexistingalloy familybutpredictivethermodynamicunderstandingofaregionofAl-MPEAphasespacethathasbeenopaquebecausethe validationpipeline(CALPHAD→atomization→AM→phaseID)hasneverbeenclosedend-to-end.

Idea

A4-yrDMREFprojectusingasinglejointpKGFNBOFNacquisitionoveraconstrainedAl-MPEAdesignspace(Al+dilute Mn/Cr/Zr/Si/Fe, ≤6 active solute elements, with the remaining critical-materials-relevant elements held at zero unless evidence supportsactivation),withCALPHADmulti-fidelitynodes(pycalphad,TC-Python/TCAL10,Thermo-CalcAMModule) asparallelpartial-evaluation/contextnodes.UA+LPBFserveastheexperimentalvalidationtier;theirparametersare nuisancevariables,notdesigntargets.Hypothesis:therapid-solidificationphasefieldsaccessibleviaUA+LPBFindilute Al-Mn-Cr-Zr-SispacecontainmetastablestrengtheningphasesthatareabsentfromcurrentCALPHADdatabasesandthat canbepredictedbyaclosedBOFN–CALPHAD–experimentloop.Thescientificdeliverableisameasured,openlypublished correctiontotheAl-MPEAphasediagraminthisregime,withtheBOFN–CALPHADloopasthemethodologicaltool.

Composition/process/propertyrecordsaredepositedtotheMaterialsDataFacilitywithcradle-to-couponprovenance.

WhyDMREF(MGIfit,multi-PIstructure)

MGIalignment:closed-loopcomputation+experiment+opendatatargetingfundamentalthermodynamics,notparameter tuningofaknownalloy.Multi-PIstructure(NSF-fundable):BYU(Baird,lead—BOFN,CALPHAD,phaseID,tensile). UnfundedU.S.collaborator:U.Utah(Spearlab—no-costLPBFcouponaccessviaestablishedMOU;notasubaward). Unfunded international collaborator:U. Toronto (J. Hattrick-Simpers, Acceleration Consortium — autonomous-experimentation methodscross-validation;supportedthroughseparateNSERCfunds,notviaNSFsubaward). 	Unfundednational-lab collaborator:CMIHubatAmes(existingletter;supply-riskcontextonly;perDMREFrulesAmesbringsitsownDOE funding).Industrialcollaborator:Thermo-Calc(CALPHADdatabaseaccess).

Preliminaryresults(FLAIMEpipeline,BYU,Apr2026)andY1scientificmilestone

AMAZEMETrePOWDERUAatomizerinstalledandoperational;UAcompositionmodificationfromAlSi10Mg+Al wire+SifeedconfirmedbySEM/EDS(thewire-to-powderpathwayisinworkingform).AlSi10MgLPBFtensilecoupon printedatU.Utah(Spearlab);elevated-temperaturetensiledataonastandardAlspecimen.WorkingpKGFNBOFN implementation;Thermo-CalcAMModulesimulationsoncandidateFLAIMEcompositions;CALPHADjobsonBYUHPC. Y1scientificmilestone(theprediction-validationexampleDMREFreviewersshouldexpect):oneclosed-loopcaseinwhichthe BOFN–CALPHADposteriorpredictsametastablephaseinadefinedAl-Mn-Cr-Zr-SiwindowthatthepublicAl-database equilibriumassessmentranksdifferently,withSEM/TEMphaseIDonaUA+LPBFcoupontoconfirmorrefutethe prediction.

ScopeseparationfrompendingDOEGenesisMissionTopic3B

Thetwoeffortsareseparatedatthetaskanddeliverablelevel,notjustbynarrative. 	Genesistasks(DOE-funded): hardware/software integration of an autonomous LPBF-AM platform;scheduler;data fabric;alloy-agnostic platform validation. DMREFtasks(NSF-funded):(T1)hypothesis-drivenCALPHAD-BOFNstudyofnon-equilibriumphaseselectionindilute Al-Mn-Cr-Zr-Sispace;(T2)thermodynamic-databasecorrectioninthatregime;(T3)openscientificdataproducts(MDF deposition,CALPHADoverlayfiles).Personneleffort,deliverables,andreportingartifactsareitemizedseparately;nobudget line,equipmentpurchase,orscheduler-developmenttaskappearsinboth.Current-and-pendingwillreflectthissplitexplicitly.

Request

AbriefreadonwhetherthisconceptappearspotentiallyresponsivetoDMREFandwhetherashortscopingcallwouldbe usefulbeforeBYU’sinternallimited-submissionprocessfortheJan–Feb2027window.

Repositoryandfullvision:https://github.com/vertical-cloud-lab.