AMMTOConceptOutline—FLAIME-CM:Domestic-FeedstockPrintableAluminumAlloys,withtheCritical MaterialsInnovationHub

PI: Sterling Baird (Brigham Young University,Materials Science & Engineering).Solicitation:DOE EERE AMMTO Multi-Topic FOA /CMIHubpartnershiptrack.Type:2–3yr,∼$1–3M(AMMTOMulti-Topicenvelope).Keywords:criticalmaterialssubstitution, LPBF,ultrasonicatomization,BOFN,CALPHAD,TRL-4.

| Feedstock | CALPHAD | pKGFNBOFN | Atomization | LPBF | PhaseID+Test |
| --- | --- | --- | --- | --- | --- |
| wire/scrap/powder | pycalphad/TC-Python/AMModule | multi-fidelityinference;GAinner | UA(BYU) | coupon | SEM/TEM,tensile |

closed-loopfeedback:phaseID+propertyupdateCALPHADpriorsandBOFNposterior

Problem

PrintableAlalloyfamiliesontheAMMTOtrajectoryrelyonCo/Ni/Sc/REEadditions;substitutiontowarddomesticallyabundant Mn/Cr/Zr/Si/Festreamsrequiresadiscoveryloopthatexplicitlycarriesafeedstocksupply-riskobjectivealongsideprintabilityand elevated-temperaturestrength.Mappingtothe2023DOECriticalMaterialsList:thesubstitutiontargetsdirectlydisplaceCo (Critical2025–2035),Sc/REE(Critical),andNi(Near-Critical)bydesign—eachcandidatealloyisevaluatedagainstaper-element exposurescoretraceabletothecurrentDOElist,scoredbytheCMIHub.TheAl–Cetrajectory(CMIHub→EckIndustries→Magna)showsaCMI-coupleddiscovery→validationpipelinecanreachcommercialdeployment;noanalogoustrajectoryyetexistsfor theAl–Mn–Cr–Zr/Al–Zr/Cu–Cr–ZrfamiliesthattheFLAIMEloopispositionedtoaddress.

Idea,deliverablecadence,andgo/no-go

A2–3yrprojectapplyingtheFLAIMEdiscoveryloop—pKGFNBOFNoverCALPHADmulti-fidelitynodes(pycalphad,TC-Python/TCAL10,AMModule),validatedbyultrasonicatomization(UA)andLPBF—toonealloyfamilyperprojectyear.Baseline: Al–Mn–Cr–ZrandAl–Zr.Stretch(Y2–Y3):Cu–Cr–ZrorBe-substitution.Hypothesis:asupply-risk-awareBOFN–CALPHADloop, withCMI-suppliedfeedstockdataandAmescross-checkproductionruns,candeliveroneTRL-4candidatealloyandanassociated UAfeedstockspecificationperprojectyear.Annualgo/no-go:(i)onevalidatedcandidatealloywithUApowderspec,LPBF parameterwindow,andtensiledataatroomandelevatedtemperature;(ii)CMI-scoredsupply-riskreductionrelativetothematched Al–Ce/Al–Ni–Scbenchmark;(iii)Amescross-checkprintconfirmingreproducibilityoutsideBYU.

Cost-shareandBABAcompliance

The20%statutorynon-federalcost-sharewillbemetbydocumentedin-kindcontributionsfromValimet(gas-atomizedcomparator powder,characterizationtime)andAMAZEMET(UAhardwareengineeringsupport),withBYUcommittingresidualcost-share throughPIsummersalaryandHPC;valuation,allowability,andPoPdocumentationwillbeaudit-ready.BABA:themanufactured articlesfundedunderthisassistancearetheUA-derivedpowderfeedstockandtheLPBF-producedAlalloycoupon/testedarticle;both areproducedintheU.S.ontheBYU-installedAMAZEMETrePOWDERandonU.S.-basedLPBFsystems.AMAZEMET’srolein cost-shareisupstreamengineeringsupport(processknow-howtransfer),notprocurementofforeign-manufacturedendproduct;no foreign-manufacturediron,steel,ormanufacturedproductisincorporatedintothedeliverablearticlestream.Ifcontractingdetermines awaiverisneeded,Valimetprovidesafullydomesticatomizationcomparatorpath.AnyEUmirror-siteengineeringinteractionis knowledgeexchangeonlyandwillbedisclosedunderresearch-securityreview.

Partnershipstructureanddownstream-adopterpathway

Funded:BYU(Baird,lead—discoveryloop,in-houseUApowdersynthesis,LPBFcoordination);CMIHub(supply-chainmodeling, supply-riskscoring,referencevalidationprintsunderseparateCMIfunding).Collaboratorwithownfunding:AmesLaboratory (cross-check production runs;per AMMTO/EERE convention Ames brings its own DOE funding rather than a budget line through BYU). Industrial:Valimet,AMAZEMET(cost-sharein-kind),Thermo-Calc(CALPHAD).Downstreamadopter(gapacknowledged, in-progress):noEck/Magna-classtransitionpartnerisyetunderletterforAl-MPEA;beforethefullwhitepaperwewillcloseone oftwopaths—LPBFservicebureauadoptingthequalifiedpowder/parameterwindow,orautomotive/aerospacetier-1evaluator analogoustoMagna’sAl-Cerole—anddisclosethenamedpartner.

Preliminaryresults(FLAIMEpipeline,BYU,Apr2026)

UAcompositionmodificationfromAlSi10Mg+Alwire+SifeedconfirmedbySEM/EDS;Thermo-CalcAMModulesimulationson candidateFLAIMEcompositions;CALPHADjobsonBYUHPC.AlSi10MgLPBFtensilebaseline;elevated-temperaturetensiledata onastandardAlspecimen;workingpKGFNBOFN;automatedpowderdispensingdemonstrated.TheAmesletterofsupportisonfile. Y1risk-retiringmilestone:oneAl-Mn-Cr-ZrorAl-ZrLPBFcouponsetproducedviatheproposedfeedstockroute,withprintability evidence,room-temperaturetensilevs.AlSi10Mg,andaper-elementexposure-scoredeltavs.anSc/REE-bearingcomparator—the closed-loopresultreviewerswillwantbeforeTRL-4claims.

ScopeseparationfrompendingDOEGenesisMissionTopic3Bandrequest

Task-levelscopeseparation.Genesis tasks (DOE / Topic 3B): alloy-agnostic platform construction — hardware integration, scheduler, datafabric,autonomousorchestration,generalplatformvalidation.AMMTOtasks(thisproposal):(T1)per-alloy-familycampaign designwithexplicitcritical-materials-listmapping;(T2)UApowderproductionandqualification;(T3)LPBFparameter-window development;(T4)elevated-temperaturemechanicaltestingtoTRL-4;(T5)CMI-scoredsupply-riskbenchmarkingandAmescross-check. Personnellaborisitemizedpertask;datapackagesandreportingartifactsarealloy-family-specificandnotGenesis-platformdeliverables. Nobudgetline,equipmentpurchase,orscheduler/data-fabrictaskappearsinboth.Request:ascopingcallwiththecognizant AMMTOPMandtheCMIHubdirector(viatheAmeslettercontact)toconfirmtheper-elementexposurescoreandTRL-4framing matchthecurrentCriticalMaterialsListbeforewedraftafullwhitepaper.

Repo:https://github.com/vertical-cloud-lab.